1049

SERVICE BULLETIN

# MAIN MOTOR TEST

# Splendide WASHER/DRYER

# MODEL WD802 PRE ”M”

To check the main motor windings you should have continuity between the following terminals:

Terminals 4, 8 & 2 (Wash Windings)

Terminals 3, 7 & 6 (Spin Windings)

Terminals 1 & 5 (Power Out)

Terminals 2 & 6 Will Test Open

Note: None of the terminals should have continuity to ground.

(4)  (8)

(3)  (7)

(2)  (6)

(1)  (5)

Motor terminal 4 is connected to 2 T on the wash timer

Motor terminal 8 is connected to 2 B on the wash timer

Motor terminal 3 is connected to 4 B on the wash timer

Motor terminal 7 is connected to 12 B on the wash timer

Motor terminal 1 is connected to pin 2 on the module board (Neutral)

1049A

SERVICE BULLETIN

MODULE BOARD TEST

# Splendide WASHER/DRYER

# MODEL WD802 PRE ”M”

Determine if the electronic circuit board is working properly.  The circuit board is located in the back of the machine above the discharge pump and has 8 pins on it.

Check for proper voltage on the circuit board:

Pin 1: neutral connection, 0 volts ac

Pin 2: power from the drive motor, 115 volts ac

power comes out of pin 1 on the motor plug (blue wire)

Pin 3: continuous power to the board, 115 volts ac

Problems: if you have no voltage at pin 2 then you have no power coming out of the motor.  If you have no voltage at pin 3 then you have no power coming from the wash timer to the board-trace the wire back and check for loose wires.

Tachometer check:

Unplug the tachometer wires from pins 7 & 8 on the board.  Check the resistance between these wires, 10-15 ohms

Problems: if the tachometer tests open the tachometer is bad or one of the wires has been damaged.  Tachometer needs to be replaced.

Main motor check:

See the attached troubleshooting guide for the main motor. See service bulletin # 1049

Additional testing:

If the circuit board has tested for the correct voltages on pins 1-3 the problem could be that the voltage is not running through the board properly.  Check this by leaving all the plugs on the board and add a jumper wire between pins 1 & 2 on the board and test the machine in the dry cycle.  If the tub rotates the board is the problem and needs to be replaced.

Technical Phone: 503.655.2563/Fax: 503.722.9202/e-mail: splendide@westlandsales.com

Web Sites:	www.splendide.com		www.westlandsales.com