Curing 300 Amp Delco Remy Alternator Leaks

Curing the oil leaks on the alternator on a DD 60 Series……

To remove the serpentine belt, there is an idler pulley in the center of the engine with an adjustment bolt (about 8 inches long) above at an angle.  Loosen this bolt about two inches allowing the pulley to move away from the belt and you can remove the belt.

The alternator pulley is removed with a 33mm socket (1 3/8 may be close).  A 1/2 air gun with 100 psi will get it off, but don't "rattle on it" for much longer than 10 seconds at a time.

Once the nut is off the pulley will slip off easily as it is a splined armature shaft. You will now see the seal plate held on with 6 bolts, remove the bolts and the seal plate will come off revealing the front bearing.

Now remove the seal collar still on the shaft and an o-ring.  Scrape the seal plate gasket off the alternator housing.  Change the seal on the plate being careful to press the seal in properly.  If the seal surface is dry apply a thin coat of grease on the lip.  Reassemble the o-ring, seal collar and seal plate and pulley, washer and nut.  Then re-install the belt and re-adjust with the idler pulley with the long adjusting bolt and you have now saved a lot of time and money.

Fred & Jeanne Hulse
Morristown,Arizona
1997 Wanderlodge WB41

Part no: 402-1410 has all the parts that Fred describes.
http://www.elreg.com/category/83.aspx

David Brady
'02 LXi, NC