Headlight Upgrade/Replacement on Early 90s Wanderlodges

The early 90 Wanderlodges used Ford headlights with modifications.  They typically get very dull and ‘fogged’ over time.  Here are notes from a member on replacing them.

The headlights are FORD F150 1986-1992. You will need two of each 
to do the replacement. Each headlight requires modification. The 
inner headlights require two mod's, 1) cut the turn signal off, and 
cut off a bracket on the opposing side. The outer headlights 
require only the outer bracket be cut off. Then you must drill a 
couple holes in each headlight and use the bracket that BB made to 
join the two together. It takes a little over two hours for each 
side to complete the work.

I got mine off ebay for $75 each including shipping, they where new 
and also included new halogen bulbs, I was very pleased with them. 
They are DOT approved. Here are the links to them:

Right headlight: http://tinyurl. com/2wnr94
Left headlight: http://tinyurl. com/2o3b6j

Forgot to mention that you will need to take the filler panel off 
between the bumper and the headlights. It has several screws on the 
outer part of the filler near the entry door and near the 
generator. You will need to slide the generator out and then you 
will see where the filler panel has a riser that goes up and 
attaches it to the area right below the electical access panel on 
the front of the coach. You will need to locate a bolt that goes up 
from that riser into the upper panel. There may also be a bolt 
that goes from the bumper up through the riser panel about half way 
between when the bumper turns and goes down the side of the coach 
and where it ends. 

Mine also had the outside temp sensor mounted to the curb side 
filler panel which I had to remove. Once you get that riser off, 
there are two bolts on each side of the headlight bracket that holds 
it to the coach.

If you have access to an upright bandsaw it make short work of the 
effort. The brackets are made out of plastic, so they can be cut 
with a hack saw if needed. I have a portable bandsaw that we used 
when racing circle track for making repairs at the race track, it 
worked great wonders on modifying them.

I have been thinking about doing two mods to the headlights. The 
lights on my coach only use one on each side for low beams, and both 
on each side for high beam. The inner light harness is missing the 
pin for the low beams, I am thinking about adding that missing pin 
to get the low beam on that light as well. The other mod would be 
to use a set of relays to drive the lights instead. I was thinking 
I would tie to a 12 volt distribution cable in the distribution 
panel above the generator and use the high/low beam signal to engage 
the relay. That way I am getting full voltage to the lights 
instead of pulling it from the light switch in the dash. 

Best of luck, if I can answer any questions about replacing them 
don't hesitate to call (i'm in the book).

Best of luck,
Michael

Michael Brundridge
92 WB40
Georgetown, TX