Dutch Radiator

This document is intended to be a reasonably detailed accounting of what's involved to convert a Bluebird Wanderlodge's radiator to what is known as a Dutch Radiator, or perhaps more descriptively, "a swing out radiator". At some point in the mid-eighties these became a standard issue.

Please post if you have suggestions that you think will make this modification work better.

A swing out radiator allows the operator to quickly access the front engine (belts/hoses)  w/o needing to spend about 3 hours of labor, and collecting 15 or so gallons of coolant, then putting it all back again, probably 4 hours of additional labor.

With a swing out radiator typically, you would remove 4 bolts to the front bumper, allowing it to swing down and out of the way on its' own hinges. You then unbolt the 15 or so 1/4'-20 bolts holding the sheet metal on in front of the radiator. Then you unbolt the two bolts on the right hand side of the radiator and swing it out of the way to access the front engine compartment. About 20 to 30 minutes.

Note that I have not done the bumper hinge portion of this project yet so that is not covered here.

Holland Moterhomes in Michigan installs this modification. I have read posts suggesting that the cost is about a thousand in parts and another thousand in labor. Another post suggested a thousand total. I e-mailed Holland Moterhomes twice requesting pricing and never received a response. If the total cost is truly a thousand, and I thought my rig could make it, I probably would have Holland do it.

Note that there may be significant year to year variations in the engine compartment so your install to some extent will vary from these directions. The rig that this document specifically details is a 1980FC31 w/a 3208N/A Caterpillar diesel engine.

I would say that this is a fairly doable project but involved. Unless you do engine compartment modifications all the time: engine work, cooling lines/fittings, welding, grinding, etc., expect the rig to be in pieces for at least a month or two. You'll probably choose to change belts/sound insulation, timing, flush coolant, replace coolant and fix the steering pump if its leaking. I also swapped out the heater hoses and that was a major pain in the rear.

The biggest challenge (up front) to this project is getting the door to open and close 90 degrees w/o anything kinking. Because of the forced hinge location, the swing radius is fairly high. (For example, my lower 2" outlet hose originally was 14" long, once the hinge was in place, fully swung I needed an addition 12" just to get it to the original radiator outlet coupling. Upon closing there is simply no where for the hose to go and it hopelessly kinks.

Initial Disassembly

Fairly strait forward, disconnect any wire leading to the front bumper, remove 4 large bumper bolts, and carefully lower the bumper (HEAVY). Remove bolts and screws on sheet metal assembly covering radiator, there may be small hosed to windshield washer nozzles that will need to be carefully pulled off (not so heavy).

Have 15 or so gallons worth of cans ready to collect coolant for disposal or reuse. Start draining at the peacock on the bottom of the radiator. Be sure the manual heater valves  are all open. For more complete drainage, there are vacuum bleeders on the header lines and additional block drainage bolts that can be opened (see your manuals).

Remove rubber hoses to radiator, some may need to be cut to free. There will be a 2" inlet hose at the top of the radiator, a 1" heater hose at the top of the radiator, and an additional 2" outlet hose at the bottom of the radiator.

There will also be two copper 3/4" outside diameter transmission coolant lines attached to the bottom of the radiator (5/8" inside diameter). These need to be cut with a small pipe cutter leaving at least 4" inches of pipe to work with both were they enter the radiator space and where they attach to the radiator itself, four cuts (recommendation). Collect a pint or so of fluid apt to drain when the lines are cut.

Find a helper, the radiator is very heavy, mine had bottom damage where someone had dropped it before. Remove the four bolts holding the radiator in place and carefully move to a safe place. You may choose to have the radiator professionally cleaned at this point. A new radiator is at least a thousand.

Radiator Mounting Bracket

Remove the bolts holding the radiator mounting bracket to the radiator. I cut my bracket centered where the bracket meets the radiator and the space to the left where the left hand side of the bracket attaches to the frame rail assembly. The farther left the better. (Someone mounted a piece of angle iron on the left two mounting bolts in their project and mounted the plate with hinges there(extreme left). This probably swing closed better than where I put the hinge, but will require more custom work in keeping spacings correct. Though the outward swing will be longer requiring longer hoses which are harder to manage, potentially.)

I then placed to galvanized gate hinges over the cut, placing the now two piece radiator mounting hinge under it such that the place had a gap along the cut the same thickness as the blade I used to cut the metal with (a portable band saw). I then welded the heck out of it with a mig welder. This resulted in an articulated mounting plate with the exact same footprint as original.

Now return mounting plate to radiator and reinstall with helper but only use the left two bolts. When closed the right hand frame rail provide a handy shelf that that side of the bracket can rest on with out the need of using the bolts for that side (when not in use of course.

On the hinge side, optionally as I did, you could add a third bolt in the unoccupied center bolt space for added strength.

Others added plate etc. to their mounting plate to make it more rigged. Mine seemed strong enough so I did not bother. There could be more than one style of plate though.

Shroud

To close the radiator I needed to make a small cut on the hinge side of the radiator shroud to allow the fan blades to clear the shroud. I've left written instructions on how to align the fan blades to both open and close the shroud w/o damage for the benefit of future owners. Basically, the fan blades are not evenly spaced, you align the bottom blade of the two widest gapped blades with the cut in the shroud, and the radiator will open or close. When closing, the radiator slightly brushes the frame rail opposite of the hinge, but no real problem.

2" Heater Hose Placement (Inlet/Outlet)

From the doghouse, the inlet at the top of the radiator was on the upper left and I moved it to the right hand side of the radiator (using the original blank plate and inlet plate). And I moved the bottom right outlet to the left hand side of the radiator using 2" automotive exhaust pipe bent at a tight 90, and then bent back at a 30 or 40 degree (see posted pictures). I read somewhere once that for best cooling the inlet/outlet should be on opposite side.

The outlet pipe was way easier to fabricate than I expected. I removed the 2" outlet flange with a band saw and grinder. Took a picture of what I wanted to and exhaust shop and they bent a piece of scrap pipe and gave it to me. I fitted it into the flange, closed the radiator and trimmed both ends until it looked like I had a reasonable fit. I mig welded the two together, mistake, I should have tack welded it together and had a welding shop close it up right. It's OK now though.

The 2" top inlet hose ended up being about 42" in length, nappa had the longest hoses I could find so I simply butted to lengths together w/the 2" pipe coupler already on the old cooling line. Everything was double clamped w/marine grade fittings (make sure the screw is stainless steele).

I think, the bottom outlet hose was 23" in length, whatever the longest size was that nappa could provide (have this hose prior to shortening the outlet fitting you will fabricate above...). These hoses are the heave ribbed and wire reinforced hoses that can be tightly bent w/o collapsing.

For the top 1" heater hose I used a 4' or 5" length of Gates Green stripe hose and the existing 90 fitting to come off the radiator to behind the alternator, followed by a long piece of the same hose that loops to the air cleaner and back to the 1" engine fitting.

Transmission Coolant Lines

These proved to be the biggest pain for me. I ended up have to go under the front engine mount with these. Simply no room to place everything when the radiator closes. Off the transmission, both the inlet and outlet I used 3/4" compression fittings (90 degree so their facing down). The compression fitting worked great, but between the compression fitting and the barbed fitting I've experienced allot of leaking, regardless of thread sealer or tightness. The barb fitting to hose does not leak but was near impossible to press on fully while working under the engine. I probably over tightened them the first time and distorted the threads. My recommendation here is, well, don't do what I did, your on your own...

I'm probable just going to thoroughly clean these assemblies and seal w/JB weld, I can always remove them at the compression nut.

Whatever you do, do have a swivel fitting at one end if possible. As with all of the hoses in this article, they need to be kink free when the radiator is closed. With the smaller hoses, w/o swivel ends, they probably will be kinking when the radiator is open (perhaps not if hydraulic hose is used).

Note that the Hydraulic shops I spoke with steered me away from simply slipping hydraulic line over a flared 3/4" pipe and double clamping it as they felt it might still pop off as these lines are so rigid. And I could not locate a flaring tool larger than 5/8".

Careful with the existing copper coolant lines as if you over bend them they will collapse.

Issues

The Gates Green Stripe hose I used is very spendy but very heavy duty, and hard as hell to press on to many of the fittings, even with soap. No leaks, resists kinking to some extent.

I now use cats new extended life coolant ($7 per gallon) and I'm experiencing leaks through (yes, through) the gasket material. While it doesn't mention this on the containers, my local caterpillar service department said this is a well known problem w/older engines (WHAT!). Sealing surfaces have to be perfect, and apparently generic, you cut it yourself gasket materials may not necessarily be coolant tight (though as mine, they may be fuel and water tight...). They could not recommend a better gasket material right off. I'll deal with this next summer as I'm tired of draining it, only a few drops a day and not much worse while driving. They said regular antifreeze has better self sealing properties.

The transmission coolant lines are now passing a bit too close to the exhaust header and I'm going to need to shield them, no real damage but a plastic spacing collar on one of the fittings started to melt. Better to deal with it now.

After the Fact

Allot of work! More so because you'll see things you want to fix while it's all open (heater hoses, belts, insulation, and who knows what, maybe adjusting those valves). I found myself staring allot, "how am I goanna deal with that"... One hours work, four hours of contemplation or worse. Also, it's all so heavy duty, even the removal of a simple bolt may take way more effort than you ever expected, up front. Perhaps this will speed things up a bit for other hobbyist's out there.

Hope this helps.

Regards,

Don Duncan

1980FC31

Beaverton, Oregon