- Updated 5.08.2008 by Curt Sprenger…

Note: Custom Instrument Panels has the senders for our gauges. Click on the link http://www.custominstrumentpanels.com/ click on gauges, scroll down and click on senders.

- Please update this document as additional information becomes available.

- TELEFLEX GAUGE AND SENDER CROSS REFERENCE

- Compiled by Leroy Eckert

I am posting what I have at the moment on these gauges. I will fill in the blanks if I get an answer from Teleflex.

Gauge - Scale	Mfg # 	BB # 	Gauge 	Sender 	Mfg #

Index 	Code  	Sender

Engine Oil - F 	9153278 	3865334 	140-320 	E 	52320S006***

Genset - PSI 	70183** 	3930815 	0-120 	V 	1506675D

Genset Temp - F	9202671 	3930807 	100-260 	AC 	52320-011 = 1/2”

52320-014 = 1/8”

Eng Water  - F 	9202671 	3930807 	100-260 	AC 	52320-011 = 1/2”

52320-014 = 1/8”

Trans Oil - F 	9153475 	?? 	140-320 	E 	52320S002

Trans Oil - F 	9202578D 	?? 	100-320* 	U 	NA

Eng Oil  - PSI 	9162070 	?? 	0-80 	C 	1A15001

Retarder - F	?? 	?? 	200-360 	? 	??

- Notes:

*Original Gauge Index, still shows the gauge but no sender

**Superceded to P/N 9179870 with(D)option

***Alternate aftermarket Stewart Warner Sender 280 EE-F works.

??Unknown at this time, # ?? possibly AC Delco

- Teleflex does not recommend using pipe extenders. Metal to metal only, no pipe dope.

Senders are classified by letter code. Select the appropriate letter code then look at the gauge index then select the P/N by NPT thread size. Have a toddy, it takes awhile.

If the senders are not correct the gauges will not read correctly. That could cause us to search for a potential problem that may not exist. Not very time or cost effective.

You will note that BB is using the same number for these gauges as Teleflex has changed the index on at least two of them. The senders received from BB on the gauges without index change should be correct even though the Teleflex P/N has changed. The senders on the gauges with an index change will not be correct from BB. They will need to be special ordered based on the code number and NPT size. I ordered mine from West Marine with a considerable dose of frustration.