FUSE & RELAY LOCATIONS

1998 thru 2003        lxi

INDEX

# I	OUTSIDE DRIVER SIDE TOP COMPARTMENT

II	FRONT DASH – LOWER FRONT

III	FRONT DASH – OVERHEAD FRONT

IV	ABOVE BED BEHIND CABINET ON PASSENGER SIDE

V         ENGINE COMPARTMENT

VI         ELECTRIC MASTER SWITCH

VII        A/T SWITCH

File:  BB FUSE PUB A

INDEX

ABS

EC-15			V

MC-12		V

ABS POWER

EC-15			V

MC-12		V

ABS warning		I-P11C1

ABS warning light

EC-15		V

MC-12		V

Air compressor-toilet	II-SK37

Air compressor-toilet	II-SK37

Air compressor relay	II-SK26

Air conditioner relays	IV-SK14

Air horn solenoid	III-SK35

Air dryer		I-P1A2

Air vents		II-SK23

A/C heat control	II-SK6

A/T systems		II-SK63

Automatic Generator start	II-SK61

Aux battery		II-SK35

Aux battery		V

Aux battery disable	II-K42

Aux battery enable	II-SK9

Aux battery solenoid  	II-K9

Back up power	I-P1B2

Bathroom, exhaust fan	IV-SK19

Bathroom, fluorescent lights, RH	III-SK43

Bedroom, alarm clock		IV-SK01

Bedroom, antenna control	IV-SK12

Bedroom, closet lights	IV-SK19

Bedroom, door lock switch	IV-SK18

Bedroom, lower fluorescent lights	IV-SK22

Bedroom, reading lights	IV-SK26

Bedroom, stereo	II-SK53

Bedroom, stereo memory	IV-SK02

Bedroom, TV antenna control	   IV-SK17

Bedroom, upper fluorescent lights	IV-SK16

Brake switch		II-K18

Burglar alarm		II-SK11

Buzzer		III-SK11

Carbon monoxide detector	III-SK48

CB radio	II-SK48

Cellular phone power		II-SK45

Cellular phone memory	II-SK13

Ceiling lights		IV-SK21

Ceiling lights (opt)	III-SK01

Clearance lights	I-P3E

Clearance lights	I-P1D3

Clearance lights	II-SK28

Clearance lights	II-K7A

Clearance lights enable	II-K7

Clearance light salute		I-P12E

Cigarette lighter – under dash	II-SK27

Cigarette lighter – dash II-SK23

Chassis A/C engine fan enable	II-K39

Clock		III-SK15

Compartment locks	II-SK46

Compartment door lock/unlock switch	III-SK52

Compass thermometer, electrical		III-SK52

Cooktop, two burner		III-SK04

Coolant level module		II-SK64

Coolant fan		I-P12D

Copilot’s window heat	II-SK21

Copilot’s window heat	II-K44

Copilot’s seat control		II-SK42

Directional flasher	I-P11D1

Directional, LH	I-P13C

Directional, RH	I-P13D

Directional lights RH & LH	III-K3

Directional flasher	I-P14B

Directional lights rear	II-SK58

Directional lights rear III-K4

Directional flasher lights	II-SK59

Directional control	II-K25 thru K33

Door lock	II-SK22

Door lock lights	II-SK35

Door lock module power/door module power	III-SK13

Door chime	III-SK30

Door entrance lock power III-SK40

Daytime Running Lights (DRL) Power	I-P1C3

Daytime Running Lights (DRL) park brake control	I-P2C

Daytime Running Lights (DRL) control	I-P11C2

Driving/Fog lights	I-P181, II-K6

Dump air valve, suspension	II-SK32

Dump air valve, tag axle	II-SK32

ECU delays	II-BAK5

Electric master switch	(filter)	II-SK10

Pilot’s Electric Seat	II-SK20

Electric heat enable	III-SK10

Electric heat enable	IV-K3

Engine compartment lights	IV-SK20

Engine compartment	I-P11D3

Engine brake lo/med	I-P13E

Engine brake hi	I-P13F

Engine brake stoplights	II-SK8

Engine brake stoplights	II-K19

Engine brake		II-K22A,K22B,K22C

Engine cold start	I-P11DS

Engine ECM ignition	V

Engine ignition	V

Entrance step system	II-K14,K15,K15A

Entrance door lock	II-K10,K11

Entrance door top lock	II-SK7, II-K21

Equalizer power	II-SK43

Fire alarm	I-P11C1

Fuel filter	V

Fuel sensor	I-P11C3

Freezer  	III-SK54

Generator actuator	II-SK33

Grill, electrical outside relay	III-SK41

Hayden fan	V

Hazard lights	II-SK58

Headlight	II-SK40 (SK31)

Headlight, power	I-P1C2

Headlight, power	II-BAK4

Headlight, lo beam	I-P3A

Headlight, lo beam	II-K35

Headlight, hi beam	I-P4A

Headlight, hi beam	II-K34

Headlight, warning	I-P12C

Headlight, alert	II-SK1

Headlight, alert	II-K8

High idle	II-K17

Horn	I-P1D1, I-P3F, II-K25

HWH jacks	loc I TOP OF COMPARTMENT-2 PL

Ignition		I-P11C1

Ignition		II-K36

Ignition		V

Ignition dropout	V

Ignition, 12V from lower front panel	III-K5

Invertors, foot of bed, inside hope chest, inside door CB

K1	IV-SK25

K3	IV-SK27

K8	III-SK05

K10	III-SK27

Keyless entry	III-SK13

Keyless module	SOL1, SOL2

Level warning		II-SK1

Leveling jacks		II-SK24

Lights, aisle	IV-SK15

Lights, copilot fluorescent	III-SK37

Lights, door locks	II-SK35

Lights, driving		I-P14A

Lights, dinette, lower fluorescent	III-SK38

Lights, dinette, ceiling		III-SK38

Lights, dinette, L	III-SK38

Lights, dining & rh living, reading, 4-light ceiling, vista	III-SK46

Lights, landing	I-P11B2

Lights, LH landing	I-P12A

Lights, RH landing	I-P12F

Lights, hall fluorescent light switch	IV-SK23

Lights, living room, 	III-SK39

Lights, co-pilot reading, window lower	III-SK45

Lights, kitchen stove	III-SK06

Lights, porch, right	III-SK37

Lights, porch, left

Lights, L/compartment lights power	I-P11B3

Lights, real halogen	IV-K1

Lights, rear parking	IV-K1

Lights switch LH landing	IV-K3

L/CMPT door lock relays I

L/CMPT door lock CB I

LPG leak detector	III-SK16

Mastered accessory	I-P11B1

Microphor toilet	III-SK08

Mirror heat	II-SK25, II-K46

Mirror switch illumination	I-P12B

Mirror remote control	II-SK25

Mirror/window time delay	II-SK7, SK47

Musical horn		II-SK50

Neutral start		I-P3C

Neutral safety		II-K24

Outside mirror remote control	II-SK25

Outside mirror heat	II-SK25

Outside compartment lights	II-SK19

Park brake	I-P1B1

Park brake interlock	I-P3B

Park lights	I-P4B

Park/taillights power	II-SK30

Park lights relay	II-K41

Parking lights	II-SK55

Pilot’s electric seat	II-SK20

Pilot’s fluorescent light	II-SK22

Power amp front		II-SK29

Power reel	Rear location

Racor filter (water)	II-SK68

Radar detector  	II-SK4

Radio, front	II-SK52

Radio, rear	II-SK53

Radio memory		II-SK38

Rear view camera	II-SK20

Recept 12vdc		II_SK23

Regulator power	V

Remote mirror control light	II-K43

Reverse	II-K23

SGM chassis A/C & heat	II-SK5

Slide out/ leveling jacks constant	I-P1D4

Slide out/ leveling jacks mastered	I-P11B4

Slide out/ level jacks brakes/neutral	I-P13B

Slide out luggage door interlock	I-P2D

Solenoid 75A ignition I

Solenoid 100A ignition I

Solenoid 100A master I

Spotlight switch ignition	III-SK34

Spot speed control	III-SK34

Step system	II-SK12

Step lights	II-SK34

Step system enable relay	II-SK44

Steering column power	I-P11A1

Stoplight	I-P1D2

Stoplight	II-SK56

Stop switch/cruise override		I-P3D

Suspension pressure power		I-P1A1

Suspension dump time delay		II-K38

System monitor panel		III-SK17

Taillights power	I-P1C1

Thermostat (comfort control center)	IV-SK10

Transmission overtemp lights		II-K20

Trip odometer		III-SK27

Turn signals RH & LH	I-P11D1

TV ignition interrupt relay	III-K10

TV antenna lights	III-SK27

TV antenna rotor	III-SK29

TV antenna up/down	III-SK32

Utility compartment lights	II-SK63

VIM ignition	I-P1A3

Water fill switch	IV-SK10

Water pump 	IV-SK27 (K3, K4)

Wiper, power		I-P1A4

Wiper, LH low/park	I-P2A

Wiper, LH high	I-P2B

Wiper, RH high	I-P2E

Wiper, RH low/park	I-P2F

Wiper, RH w/s	II-SK2

Wiper, LH w/s		II-SK3

Wiper, w/s column power	II-SK51

Wiper, RH w/s lo	II-K1

Wiper, LH w/s lo	II-K2

Wiper, RH w/s hi	II-K3

Wiper, LH w/s hi	II-K4

Wiper, washer fluid	II-SK2

+12 accessory		I-P11D2

12V filter coil		II-SK35

12V filter		II-SK60

12V filter		II-K37

File: BB FUSE PUB B

RELAY & FUSE LOCATIONS

LOCATION 1

# OUTSIDE DRIVER SIDE TOP COMPARTMENT

## FUSE CHART P1

P1A1	Suspension Pressure Power

P1A2	Air Dryer

P1A3	Vim Ign.

P1A4	Wiper Motor Power

P1B1	Park Brake

P1B2	Backup Lights

P1B3	Slideout/Level Jacks Ign.

P1B4	Spare

P1C1	Park/Tail Lights Control

P1C2	Headlight Power

P1C3	DRL Constant

P1C4	Suspension Dump

P1D1	Horn

P1D2	Stop Light

P1D3	Clearance Lights

P1D4	Slide Out/Leveling Jacks Constant

### RELAY CHART P2

P2A	LH Wiper Low/Park

P2B	LH Wiper High

P2C	DRL Park Brake Control

P2D	Slide Out Luggage Door Interlock

P2E	RH Wiper High

P2F	RH Wiper Low/Park

### RELAY CHART P3

P3A	Headlight Lo Beam

P3B	Park Brake Interlock

P3C	Neutral Start

P3D	Stop Switch/Cruise Override

P3E	Clearance Lights

P3F	Horn

### COMPONENT CHART P4

P4A	Headlight Hi Beam

P4B	Park Lights

### OUTSIDE DRIVER SIDE TOP COMPARTMENT

### FUSE CHART P 11

P11A1	  Dir/Wiper Cntl.

P11A2	  Spare

P11A3	  Spare

P11A4	  Spare

P11B1	  Mastered Accessory

P11B2	  Landing Light

P11B3	  L/Compartment Lights Power

P11B4

P11C1	  Ignition

P11C2	  DRL Control

P11C3   Fuel Sensor

P11C4	  H/Vac.

P11D1	  Dir/Hazard Flasher

P11D2	  +12 Accessory

P11D3	  Engine Compartment, Cold Start

P11D4	  Spare

### RELAY CHART P12

P12A	  LH Landing Lights

P12B	  Mirror Switch Illumination

P12C	  Headlight Warning

P12D	  Coolant Fan

P12E	  Clearance Light Salute

P12F	  RH Landing Lights

### RELAY CHART P13

P13A	  Stop Light

P13B	  Slide Out/Level Jacks Brake/Neutral

P13C	  LH Directional

P13D	  RH Directional

P13E	  Engine Brake Lo/Med

P13F	  Engine Brake HI

### COMPONENT CHART P14

P14A	  Driving Lights

P14B	  Directional Flasher

2 UNIDENTIFIED 7.5AMP FUSES:  HWH Jacks

File:BB FUSE PUB C

RELAY & FUSE LOCATIONS

LOCATION 11

# FRONT DASH – LOWER FRONT

## FUSE/BREAKER CIRCUITS

## SK1	Ign., Inst. PNL

SK2	Wiper Washer Fluid

SK3	Buzzer Ignition

SK4	Radar Detector

SK5	Step System

SK6	A/C Heat Control

SK7	Mirror/Window Time Delay;  Entrance Door Top Lock

SK8	Burglar Alarm Ign.

SK9	Aux. Battery Enable

SK10	Electric Master Switch (Filtered)

SK11	Burglar Alarm

SK12	Step System Power

SK13	Cellular Phone Memory

SK14	Spare

SK15	Spare

SK16	Spare

SK17	Spare

SK18	Spare

SK19	Spare

SK20	Rear View Camera;  Pilot’s Electric Seat

SK21	Spare

SK22	Door Lock

SK23	Recept. 12VDC/Air Vents

SK24	Spare

SK25	Spare

SK26	Auxiliary Air Compressor Relay

SK27	Cig. Lighter (CB Door)

SK28	Master Filter Relay

SK29	Power Amp., Frt.

SK30	Sub-woofer Amp.

SK31	Headlights (In location SK40)

SK32	Privacy Sw. Power

SK33	Generator Actuator

SK34	Step System Master

SK35	Dash LED’s

SK36	Spare

SK37	Air Compressor for Toilet

SK38	Radio Memory

SK39	Spare

# FRONT DASH – LOWER FRONT

SK40	(SK31 - Headlight)

SK41	Spare

SK42	Spare

SK43	Equalizer Power

SK44	Step System Enable Relay

SK45	Cellular Phone Power

SK46	Spare

SK47	Side View Camera

SK48	CB Radio

SK49	Spare

SK50	Musical Horn

SK51	Spare

SK52	Tuner/Cassette

SK53	Bedroom Stereo

SK54	Spare

SK55	Spare

SK56	Spare Lights

SK57	Spare

SK58	Spare

SK59	Microphor Comp.

SK60	12V Filter

SK61	Automatic Generator Start

SK62	Utility Compartment Lights

SK63	A/T System

BAK4	Headlights

BAK5	ECU Delays

BAK6	Hi Beam Indicator/Driving Lights

### RELAY CIRCUITS

K1	RH W/S Wiper LO

K2	LH W/S Wiper LO

K3	RH W/S Wiper HI

K4	LH W/S Wiper HI

K6	Driving/Fog Lights

K7	Clearance Lights Enable

FRONT DASH – LOWER FRONT

K7A	Clearance Lights

K8	Headlight Alert

K9	Auxiliary Battery Solenoid

K10	Entrance Door Lock

K11	Entrance Door Lock

K14	Entrance Step System

K15	Entrance Step System

K15A	Entrance Step System

K17	HI Idle

K18	Brake Switch

K19	Engine Brake Stoplights

K20	Tran Over Temp Lights

K21	Entrance Door Top Lock

K22A	Engine Brake

K22B	Engine Brake

K22C	Engine Brake

K23	Reverse

K24	Neutral Safety

K25	Electric Horn

K26	Directional Control

K27	Directional Control

K28	Directional Control

K29	Directional Control

K30	Directional Control

K31	Directional Control

K32	Directional Control

K33	Directional Control

K34	Head Lights HI Beam

K35	Head Lights LO Beam

K36	Ignition

K37	12V Filter

K38	Suspension Dump Time Delay

K39	Chassis A/C Engine Fan Enable

K41	Park Lights Relay

K42	Auxiliary Battery Disable

K43	Remote Mirror Control Light

K44	Copilot’s  Window Heat

K45	Pilot’s Window Heat

K46	Mirror Heat

K47	Mirror/Window Time Delay

#### File:BB FUSE PUB C

RELAY & FUSE LOCATIONS

LOCATION III

# FRONT DASH – OVERHEAD FRONT

SK01	Ceiling Lights (Optional)

SK02	Florescent Lights, Kitchen Overhead - Reading Lights, Living Room

SK04	Two Burner Cook Top

SK05	Spare

SK06	Kitchen Stove Light

SK08	Microphor Toilet

SK09	Spare

SK10	Electric Heat Enable

SK11	Buzzer

SK12	To SK1-9 Bus Rear LD. Control

SK13	Door Lock Module Power; Door Module Power

SK14	Lower Front Compartment Panel Fuse Blk.

SK15	Clock

SK16	LPG Leak Detector

SK17	Systems Monitor Panel

SK18	Spare

SK19	Spare

SK20	Spare

SK21	Spare

SK22	Spare

SK23	Spare

SK24	Spare

SK25	Spare

SK26	Spare

SK27	K10; Trip Odometer; TV Antenna Lights, Co-Pilot PNL Ign.

SK28	Spare

SK29	TV Antenna Rotor

SK30	Door Chime

SK31	Spare

SK32	TV Antenna Up/Down

SK33	Florescent Kitchen

SK34	Spot Light Switch Ignition, Spot Speed Control

SK35	Air Horn Solenoid K10

SK36	Spare

SK37	Porch Light, Right

SK38	Dinette Lower Florescent/Dining (4) Lights Ceiling Fixture;  L-Dinette

SK39	Living Room Florescent Lights, Upper

SK40	Entrance Door Lock Power

SK41	Outside Electric Grill Relay, Window, Vistas, Living Rm.

FRONT DASH – OVERHEAD FRONT

SK42	Co-Pilot Seat Controls

SK43	Bathroom Florescent Lights, RH

SK44	Dinette Table Motor

SK45	Co-Pilot Reading, Window Lower

SK46	Reading Lights, Dining & RH Living Room; 4-Light Ceiling Fixture;  Vista Lights

SK47	Ceiling Spotters

SK48	Electric Compass/Thermometer

SK49	Spare

SK50	Spare

SK51	Spare

SK52	Compartment Door Lock/Unlock Switch

SK53	Spare

SK54	Freezer

SOL1	Keyless Module

SOL2	Keyless Module

K3	RH & LH Directional Lights

K4	RH & LH Turn Signals; Rear Directional Light (Power)

K5	Ignition 12V From Lower Front Compartment Panel

K10	TV Ignition Interrupt Relay

#### File:BB FUSE PUB E

RELAY & FUSE LOCATIONS

LOCATION IV

LOCATED ABOVE BED, BEHIND CABINET ON PASSENGER SIDE:

# SK01	Bedroom Alarm Clock

SK02	Bedroom Stereo Memory

SK03	Thermostat, Comfort Control Center (Terminal 21)

SK04	Spare

SK05	Spare

SK06	Spare

SK07	Spare

SK08	Spare

SK09	Spare

SK10	Water Fill Switch

SK11	Upper Bedroom Fluorescent Lights

SK12	Bedroom Antenna Control

SK13	Spare

SK14	Air Conditioner Relays

SK15	Aisle Lights

SK16	Bedroom Upper Fluorescent Lights

SK17	Bedroom TV Antenna Motor

SK18	Bedroom Door Lock Switch

SK19	Bathroom Exhaust Fan; Bedroom Closet Lights

SK20	Engine Compartment Lights

SK21	Ceiling Lights

SK22	Lower Bedroom Fluorescent Lights

SK23	Hall Fluorescent Light Switch

SK24	Terminal 22

SK25	K1

SK26	Bedroom Reading Lights

SK27	Water Pump (K3, K4)

Power Reel

K1	Real Halogen, Rear Parking Lights

K3	Bathroom Water Pump Switch, Electric Heat Enable, LH Landing Light Switch

K4	Water Pump Switch Power

#### File: BB FUSE PUB F

RELAY & FUSE LOCATIONS

LOCATION V

ENGINE COMPARTMENT

MICRO RELAYS

| IGNITION DROPOUT | ABS MC-12 POWER | ABS EC-15 POWER |
| --- | --- | --- |
| FUEL FILTER | ENGINE ECM IGNITION | ABS EC-15 |

FUSES

| ENGINE ECM IGNITION 5A |  |  |  |
| --- | --- | --- | --- |
|  |  | HAYDEN FAN 20A | HAYDEN FAN 20A |
| ABS EC-15 30A | ABS MC-12 15A | FUEL FILTER 20A |  |
|  | IGNITION 10A | REG POWER 20A | ENGINE IGN 15A |

|  |  | ABS EC-15 WARNING LIGHT |
| --- | --- | --- |
| IGNITION | AUX BATTERY | ABS MC-12 WARNING LIGHT |

MICRO RELAYS

RELAYS

| HAYDEN FAN | REGULATOR POWER |
| --- | --- |

#### File: BB FUSE PUB G